ASSAM AI ROADWATCH

AI-Based Assam Road Health & Maintenance Prioritisation Platform

Concept / Technical Proposal Document

# 1. Executive Summary

Assam AI RoadWatch is an integrated road-condition intelligence platform combining AI-based road defect detection, GIS/GPS geolocation, citizen reporting, PWD vehicle surveys, engineer verification, road-health monitoring and maintenance prioritisation. The concept supports—not replaces—engineering judgement and departmental approval.

# 2. Core Concept

The platform should not be limited to a citizen pothole complaint application. Citizen reports are one input source. The core capability is continuous, repeatable road-condition surveying using PWD vehicles and/or survey cameras, followed by AI detection, geospatial mapping, historical comparison and maintenance workflow.

# 3. Proposed Data Sources

| Source | Captured Information | Purpose |
| --- | --- | --- |
| PWD survey vehicle | Video/images, GPS, timestamp, optional IMU | Systematic road inspection |
| Citizen app | Photo/video, GPS, timestamp, optional description | Additional evidence and local reporting |
| PWD engineer | Inspection result, severity/verification, remarks | Official verification and workflow |

# 4. End-to-End Workflow

# 5. AI Detection

Potholes

Longitudinal/transverse cracks

Rutting or surface deformation

Patches / repaired areas

Other configured road-surface distress categories

Each detection should retain model confidence, image/frame evidence, GPS coordinates, survey timestamp and model/version information. Confidence should be used as a review signal, not as an automatic engineering decision.

# 6. Citizen Reporting Workflow

Citizen opens “Report Road Damage”.

Camera captures road evidence.

Mobile device requests location permission and records GPS.

AI performs a preliminary image-quality and defect check.

The report is stored with evidence and routed for review.

Nearby/duplicate reports can be grouped with an existing defect record.

# 7. Duplicate and Same-Defect Tracking

Use GPS proximity, road segment, time window and visual similarity to associate multiple reports with one defect record. This supports a defect life cycle such as PTH-102: first detection → repeat detection → repair → post-repair verification.

# 8. Road Health and Change Tracking

Roads can be divided into configurable segments. Segment indicators can include defect count, defect density, severity distribution, historical condition and rate of change. A Road Health Index may be implemented as a configurable engineering-supported indicator; its formula should be validated with PWD rather than presented as an official standard.

| Road Segment | Defects | Illustrative Health | Illustrative Priority |
| --- | --- | --- | --- |
| NH-27 Segment A | 5 | 86/100 | Low |
| NH-27 Segment B | 21 | 61/100 | Medium |
| NH-27 Segment C | 48 | 32/100 | High |
| NH-27 Segment D | 71 | 18/100 | Critical |

Illustrative values only; not official PWD scores.

# 9. Maintenance Prioritisation Workflow

AI Detection → GIS Location → Verified Severity → Historical Deterioration → Road Importance → Priority → PWD Verification → Maintenance → Re-survey.

# 10. Evidence Pack

Each defect should provide road/segment, coordinates, detection date, defect type, confidence, image/video evidence, previous observations, citizen reports, PWD inspections, verification status, maintenance status and audit history.

# 11. Four Main Portals

| Portal | Main Functions |
| --- | --- |
| Citizen App | Report damage, capture evidence, GPS, report tracking |
| PWD Engineer App | Road survey, inspection, verification, field remarks |
| PWD Admin Dashboard | GIS map, defects, road health, priorities, maintenance reports |
| Super Admin | Users, divisions/circles/roads, configuration, AI model settings, audit logs |

# 12. Suggested Technical Architecture

Frontend: React / Next.js web dashboard; Android/iOS or cross-platform citizen/engineer app.

Backend: Spring Boot or equivalent REST service.

Database: PostgreSQL + PostGIS (recommended for geospatial workloads) or an approved government database stack.

Media: S3-compatible object storage for evidence images/videos.

AI: Python inference service using a trained object-detection/segmentation model.

Maps: Government-approved GIS basemap/services; road inventory should remain the authoritative source.

Security: RBAC, audit logs, encrypted transport, controlled media access and secure API authentication.

# 13. Core Data Entities

Road

Road Segment

Defect

Survey

Survey Evidence

Citizen Report

Defect Observation

Maintenance Action

User / Role

AI Model Version

Audit Log

# 14. Recommended MVP

Citizen photo + GPS reporting

PWD vehicle image/video upload with GPS

AI pothole and crack detection

GIS defect map

Engineer verification workflow

Duplicate report grouping

Road-segment dashboard

Basic historical comparison

Evidence pack and downloadable reports

# 15. Future Enhancements

Automated frame extraction from survey video

Same-defect visual matching

Pavement distress severity estimation

Offline-first field inspection mode

Road-risk analytics

Integration with existing road inventory / asset-management systems

Maintenance work-order and contractor workflow

Before/after repair verification

Model monitoring and periodic re-training using verified field data

# 16. Key Benefits

Faster and more consistent road-condition surveillance

Geospatially traceable evidence

Single view of citizen and departmental observations

Historical deterioration tracking

Reduced duplicate reporting

Evidence-supported maintenance planning

Better visibility for engineers and administrators

Foundation for continuous road asset management

# 17. Governance and Engineering Controls

The system should preserve human review for official defect verification, severity confirmation, maintenance prioritisation and repair closure. AI outputs should be traceable to the model version and source evidence. Road inventory and engineering classifications should follow departmental authoritative data and approved standards.

# 18. Proposed Demonstration Scenario

For a challenge demo, show one survey route with several defects, one citizen report near a detected pothole, automatic duplicate grouping, a GIS map, a defect evidence pack, a historical change view, engineer verification, and a post-maintenance re-survey.

# 19. Conclusion

Assam AI RoadWatch positions AI as an intelligence layer over road inspection and asset-management workflows. Its distinguishing value is the combination of systematic survey data, citizen evidence, GIS, repeat observations, defect life-cycle tracking and engineer-controlled maintenance planning.

Prepared as a concept / technical proposal — September 2026